《计算机组成原理》实验报告

| 年级、专业、班级 |  | 计算机科学与技术5班、6班 |  |  | 姓名 与学号 | 胡栋月20181778 彭燃20181719 |
| --- | --- | --- | --- | --- | --- | --- |
| 实验题目 | 实验三 简易单周期CPU实验 |  |  |  |  |  |
| 实验时间 | 2020年 |  | 实验地点 | DS1410 |  |  |
| 实验成绩 |  |  | 实验性质 | □验证性 设计性 □综合性 |  |  |
| 教师评价： □算法/实验过程正确； □源程序/实验内容提交 □程序结构/实验步骤合理； □实验结果正确； □语法、语义正确； □报告规范； 其他： 评价教师签名： |  |  |  |  |  |  |
| 一、实验目的 （1）掌握不同类型指令在数据通路中的执行路径。 （2）掌握Vivado仿真方式。 |  |  |  |  |  |  |
| 二、实验项目内容 1 实验内容 本实验包含以下模块： (1) Datapath，其中主要包含 alu，PC，adder、mux2、signext（扩展）、sl2（移位） (2) Controller(实验二已完成)，其中包含两部分，分别为maindec，aludec。 (3) 指令存储器instruction_rom，数据存储器data_ram（在Mems项目内，详见附带的项目文件）。使用Block Memory Generator IP构造。加载coe文件。 (4) mips将datapath与controller连接起来，而top文件将两个存储器与mips连接起来。连接示意图如下： 2 实验设计 2.1 数据通路 2.1.1 功能描述 数据通路包括寄存器堆、ALU、加法器、多路选择器、移位器与符号拓展模块。负责根据传入的信号及数据进行运算，同时可以将运算结果存入寄存器或输出到存储器中。 2.1.2 接口定义 接口信号名; 方向; 宽度; 含义. clk; input; 1-bit; 时钟信号. rst; input; 1-bit; 刷新信号. instr; input; 32-bit; 指令. ram_read_data; input; 32-bit; 数据存储器传入的数据. MemtoReg; output; 1-bit; 略. MemWrite; output; 1-bit; 略. ALUSrc; output; 1-bit; 略. RegDst; output; 1-bit; 略. RegWrite; output; 1-bit; 略. PCSrc; output; 1-bit; 略. Jump; output; 1-bit; 略. ALUControl; output; 3-bit; 略. |
| 三、实验过程或算法（源程序） datapath代码如下： module datapath( input clk, input rst, input [31:0]instr, input [31:0]ram_read_data, input MemtoReg, input MemWrite, input ALUSrc, input RegDst, input RegWrite, input PCSrc, input Jump, input [2:0]ALUControl, output [31:0] ALUOut, output [31:0] WriteData, output [31:0] address, output zero //,output [31:0]test_jump_address, //output [31:0]test_plus_4_address, //output [31:0]test_branch_address, //output [31:0]test_shifted_address, //output [31:0]test_immea, //output [31:0]test_reg_out_1 ); wire [4:0]rs; wire [4:0]rt; wire [4:0]rd; wire [15:0]immeadiate; wire [25:0]jump_immea; assign rs=instr[25:21]; assign rt=instr[20:16]; assign rd=instr[15:11]; assign immeadiate=instr[15:0]; assign jump_immea=instr[25:0]; wire [4:0]reg_write_destination; wire [31:0]reg_data_1; wire [31:0]reg_data_2; mux2 #(5)regdst(rt,rd,RegDst,reg_write_destination); wire [31:0]reg_write_data; mux2 #(32) memtoreg (ALUOut , ram_read_data, MemtoReg, reg_write_data ) ; regfile regfile( .clk(clk), .we3(RegWrite), .ra1(rs),.ra2(rt),.wa3(reg_write_destination), .wd3(reg_write_data), .rd1(reg_data_1),.rd2(reg_data_2) ); assign WriteData=reg_data_2; wire [31:0]extented_immea; signext signext(immeadiate,extented_immea); wire [31:0] alu_datain_2; mux2 #(32) alusrc(reg_data_2,extented_immea,ALUSrc,alu_datain_2); alu alu(reg_data_1,alu_datain_2,ALUControl,ALUOut,zero); //assign test_immea=extented_immea; //assign test_reg_out_1=reg_data_1; wire [31:0] shifted_extented_immea; sl2 sl2_0(extented_immea,shifted_extented_immea); wire [31:0] next_address; pc pc( .next_address(next_address), .clk(clk), .rst(rst), .address(address) ); wire [31:0] plus_4_address; adder plus_4_adder(address,32'h4,plus_4_address); wire [31:0] branch_address; adder branch_adder(shifted_extented_immea, plus_4_address,branch_address); wire [31:0] tmp_address; mux2 #(32)pcsrc(plus_4_address,branch_address,PCSrc,tmp_address); wire [31:0] shifted_jump_immea; sl2 sl2_1({6'h0,jump_immea},shifted_jump_immea); wire [31:0] jump_address; assign jump_address={plus_4_address[31:28],shifted_jump_immea[27:0]}; //assign test_jump_address=jump_address; mux2 #(32)jump(tmp_address,jump_address,Jump,next_address); //assign test_plus_4_address=plus_4_address; //assign test_branch_address=branch_address; //assign test_shifted_address=shifted_extented_immea; endmodule top代码如下： module top( input wire clk,rst, output wire [31:0] WriteData,ALUOut, output wire MemWrite, //output wire [31:0] test_instr, //output wire [31:0] test_ram_read_data, //output wire [31:0] test_address, /*output test_memtoreg,test_alusrc,test_regdst,test_regwrite,test_jump,test_pcsrc,test_zero,*. //output [2:0]test_alucontrol, //output [31:0]test_jump_address, //output [31:0]test_plus_4_address, //output [31:0]test_branch_address, //output [31:0]test_shifted_address, //output [31:0]test_immea, //output [31:0]test_reg_out_1 ); reg [2:0]count=2'h0; reg clk0=0; always @(posedge clk) begin if(count==7) begin count<=0; clk0<=~clk0; end else begin count<=count+1; end end wire[31:0] instr_address,instr,ram_read_data; mips mips(clk0, rst, instr, ram_read_data, MemWrite, instr_address, ALUOut, WriteData, /*test_memtoreg,test_alusrc,test_regdst,test_regwrite,test_jump,test_pcsrc,test_zero,*/ /*test_alucontrol, test_jump_address, test_plus_4_address, test_branch_address, test_shifted_address, test_immea, test_reg_out_1*/ ); instruction_rom imem(.clka(clk), .ena(1'b1), .addra(instr_address), .douta(instr) ); data_ram dmem(.clka(clk), .ena(MemWrite), .addra(ALUOut), .dina(WriteData), .douta(ram_read_data) ); //assign test_instr=instr; //assign test_ram_read_data=ram_read_data; //assign test_address=instr_address; endmodule 其中，灰色注释部分是为了仿真时方便观察而添加的中间变量。 红色部分的代码为时钟分频，由于本实验所使用的Vivado版本过于老旧，导致存储器的延迟达两个周期，为了保证取指和其余操作大致发生在同一周期内，所以必须时钟分频，让mips模块用较慢的时钟信号。 |  |  |  |  |  |  |
| 四、实验结果及分析和（或）源程序调试过程 上面两图是另写的仿真（参见sim_for_top.v文件）的波形图，test_instr为指令（16进制），对比下图指令中的机器码（红色方框圈起来的指令为执行了的指令）可知，两个beq指令和j指令都跳转正确。 具体每条指令的控制信号这里不方便展示，如果想查看请参考附带项目文件中的波形图文件sim_for_top_behav.wcfg。 下图为已给的testbench的运行结果，显示Simuliation succeeded，仿真成功。 |  |  |  |  |  |  |